# Subnetting Worksheet  - Traditional (Similar to TSP03)

Given IPv4 Address to Subnet : 10.0.0.0/8

- Understand requirements. Are you subnetting based on the number of subnets or based on the number of hosts?

  - If based on subnets, i) use: 2s > NUMBER_OF_SUBNETS, ii) solve for s, iii) value of s = number of subnet bits in the CAM.

    - Number of Subnets Required: 12, 2s > 12  value of s: 4

  - If based on hosts, use: 2h - 2 >= NUMBER_OF_HOSTS, ii) solve for h, iii) value of h = number of host bits in the CAM.

    - Number of Usable Hosts Required: _______________, 2h - 2 >= _______________, value of h: __________

- Custom Address Map (CAM). Use class of given address & number of h or s bits in step 1: N.sssshhhh.H.H

- Identify the Interval and Transition Octet. INTERVAL: 16  TRANSITION OCTET (TO) - 2nd, 3rd, or 4th:  2nd

  - The INTERVAL (INT) is the value of the last/rightmost subnet bit. The TO is the octet where the last/rightmost subnet bit resides.

- Prefix, Custom Subnet Mask, and Wildcard Mask: PREFIX: /12 CSM: 255.240.0.0  WCM: 0.15.255.255

- For the 1st subnet/network address, use the given IP address and specify the prefix length from step 4.

- To determine the 2nd subnet (NTWK2), add the INTERVAL to the value in the TO of the 1st subnet (NTWK1)

- To determine the 3nd subnet (NTWK3), add the INTERVAL to the value in the TO of the 2nd subnet (NTWK2). Continue this pattern…

| Nth Subnet | Interval & Transition Octet (TO) | Network Address / Prefix Length First Usable Last Usable Broadcast Address |  | Address Determination Process TO: Transition Octet = Octet containing the last/rightmost subnet bit TOV: Transition Octet Value = value in the Transition Octet INT = Interval |
| --- | --- | --- | --- | --- |
| 1st Subnet | 16, 2nd | NTWK1 | 10.0.0.0/12 | Use the original address given and apply the prefix from step 4. |
|  |  | FIRST | 10.0.0.1 | Add 1 to the fourth octet of NTWK1 |
|  |  | LAST | 10.15.255.254 | Subtract 1 from the fourth octet of BCST1 |
|  |  | BCST1 | 10.15.255.255 | Add the wildcard mask to NTWK1 |
| 2nd Subnet | 16, 2nd | NTWK2 | 10.16.0.0/12 | Copy the classful octets, TOV2 = TOV1 + INT, turn off host bits, apply prefix (step 4) |
|  |  | FIRST | 10.16.0.1 | Add 1 to the fourth octet of NTWK2 |
|  |  | LAST | 10.31.255.254 | Subtract 1 from the fourth octet of BCST2 |
|  |  | BCST2 | 10.31.255.255 | Add the wildcard mask to NTWK2 |
| 3rd Subnet | 16, 2nd | NTWK3 | 10.32.0.0/12 | Copy the classful octets, TOV3 = TOV2 + INT, turn off host bits, apply prefix (step 4) |
|  |  | FIRST | 10.32.0.1 | Add 1 to fourth octet of NTWK3. |
|  |  | LAST | 10.47.255.254 | Subtract 1 from fourth octet of BCST3. |
|  |  | BCST3 | 10.47.255.255 | Add the wildcard mask to NTWK3. |
| 4th Subnet | 16, 2nd | NTWK4 | 10.48.0.0/12 | Copy the classful octets, TOV4 = TOV3 + INT, turn off host bits, apply prefix (step 4) |
|  |  | FIRST | 10.48.0.1 | Add 1 to fourth octet of NTWK4. |
|  |  | LAST | 10.63.255.254 | Subtract 1 from fourth octet of BCST4. |
|  |  | BCST4 | 10.63.255.255 | Add the wildcard mask to NTWK4. |

# Subnetting Worksheet  - Traditional (Similar to TSP03)

| Nth Subnet | Interval/ [Transition] Octet | Network Address / Prefix Length First Usable Last Usable Broadcast Address |  | Address Determination Process Transition Octet: Octet containing the last/rightmost subnet bit. TOV: Value in the Transition Octet. TOVG: Value in the Transition Octet of original Given address. |
| --- | --- | --- | --- | --- |
| 5th Subnet | 16, 2nd | NTWK5 | 10.64.0.0/12 | Copy the classful octets , TOV5 = TOV4 + INT, turn off host bits, apply prefix (step 4) |
|  |  | FIRST | 10.64.0.1 | Add 1 to fourth octet of NTWK5 |
|  |  | LAST | 10.79.255.254 | Subtract 1 from fourth octet of BCST5 |
|  |  | BCST5 | 10.79.255.255 | Add the wildcard mask to NTWK5 |
| 6th Subnet | 16, 2nd | NTWK6 | 10.80.0.0/12 | Copy the classful octets , TOV6 = TOV5 + INT, turn off host bits, apply prefix (step 4) |
|  |  | FIRST | 10.80.0.1 | Add 1 to fourth octet of NTWK6 |
|  |  | LAST | 10.95.255.254 | Subtract 1 from fourth octet of BCST6 |
|  |  | BCST6 | 10.95.255.255 | Add the wildcard mask to NTWK6 |
| 7th Subnet | 16, 2nd | NTWK7 | 10.96.0.0/12 | Copy the classful octets , TOV7 = TOV6 + INT, turn off host bits, apply prefix (step 4) |
|  |  | FIRST | 10.96.0.1 | Add 1 to fourth octet of NTWK7 |
|  |  | LAST | 10.111.255.254 | Subtract 1 from fourth octet of BCST7 |
|  |  | BCST7 | 10.111.255.255 | Add the wildcard mask to NTWK7 |
| 8th Subnet | 16, 2nd | NTWK8 | 10.112.0.0/12 | Copy the classful octets , TOV8 = TOV7 + INT, turn off host bits, apply prefix (step 4) |
|  |  | FIRST | 10.112.0.1 | Add 1 to fourth octet of NTWK8 |
|  |  | LAST | 10.127.255.254 | Subtract 1 from fourth octet of BCST8 |
|  |  | BCST8 | 10.127.255.255 | Add the wildcard mask to NTWK8 |
| 9th Subnet | 16, 2nd | NTWK9 | 10.128.0.0/12 | Copy the classful octets , TOV9 = TOV8 + INT, turn off host bits, apply prefix (step 4) |
|  |  | FIRST | 10.128.0.1 | Add 1 to fourth octet of NTWK9 |
|  |  | LAST | 10.143.255.254 | Subtract 1 from fourth octet of BCST9 |
|  |  | BCST9 | 10.143.255.255 | Add the wildcard mask to NTWK9 |
| 10th Subnet | 16, 2nd | NTWK10 | 10.144.0.0/12 | Copy the classful octets, TOV10 = TOV9 + INT, turn off host bits, apply prefix (step 4) |
|  |  | FIRST | 10.144.0.1 | Add 1 to fourth octet of NTWK10 |
|  |  | LAST | 10.159.255.254 | Subtract 1 from fourth octet of BCST10 |
|  |  | BCST10 | 10.159.255.255 | Add the wildcard mask to NTWK10 |
| 11th Subnet | 16, 2nd | NTWK11 | 10.160.0.0/12 | Copy the classful octets, TOV11 = TOV10 + INT, turn off host bits, apply prefix (step 4) |
|  |  | FIRST | 10.160.0.1 | Add 1 to fourth octet of NTWK11 |
|  |  | LAST | 10.175.255.254 | Subtract 1 from fourth octet of BCST11 |
|  |  | BCST11 | 10.175.255.255 | Add the wildcard mask to NTWK11 |